Using the Nand Flash Image Generator Program (preliminary version)

- The default storage parameters are for hynix 64 megabyte small page slc flash.

Adjust parameters for other nand flash types.

- Select "Flash boot code from file:" :

* Click the "..." button

* Select an .img file from your hard disk and click "Open"

* (Note: To test and develop your application code you can choose "No boot code in flash - must use emulator". This way you can use the serial port emulator to load code to VS1000 for testing.)

- Go to your OGG file directory (c:\_ogg in example picture)

- Double-click the .ogg files You wish to burn to flash. The files will be played in the order shown.

- Click "Write Image File"

- Select a good name for the flash-prommer image file

- Close the image generator program and open the (for example Elnec) programmer program.

- Burn the image file to nand flash with flash-prommer with the parameters:

Select the generated binary file (e.g. vs1000.img) and program.